# 1. HUB

## 🟢 বাংলা Concept

Hub হলো একটি Basic Networking Device যা একাধিক Computer-কে সংযুক্ত করে। এটি কোনো Data Check করে না; বরং একটি Port-এ Data পেলে সব Port-এ পাঠিয়ে দেয় (Broadcast)।

## ✅ Exam Ready Answer (English)

A Hub is a basic networking device that connects multiple computers in a LAN. It broadcasts incoming data to all connected devices without checking the destination address.

### Easy Example

Suppose PC-1 sends data to PC-2. A Hub sends the data to PC-2, PC-3, and PC-4. Only PC-2 accepts it; others ignore it.

### Key Points

- Layer: Physical Layer (Layer 1)

- Intelligent Device: ❌ No

- Speed: Slow

- Data Transmission: Broadcast

# 2. SWITCH ⭐⭐⭐ (Most Important)

## 🟢 বাংলা Concept

Switch-ও একাধিক Computer-কে সংযুক্ত করে, কিন্তু এটি MAC Address দেখে শুধুমাত্র সঠিক Device-এ Data পাঠায়।

## ✅ Exam Ready Answer (English)

A Switch is a networking device that connects multiple devices in a LAN and forwards data only to the intended destination using MAC addresses.

### Easy Example

Suppose PC-1 sends data to PC-2.

The Switch checks the MAC address and sends the data only to PC-2.

### Key Points

- Layer: Data Link Layer (Layer 2)

- Intelligent Device: ✅ Yes

- Speed: Fast

- Data Transmission: Unicast

### Used In

Banks, Offices, Universities

# 3. ROUTER ⭐⭐⭐

## 🟢 বাংলা Concept

Router দুটি বা ততোধিক Different Network-কে সংযুক্ত করে এবং IP Address ব্যবহার করে Data-এর সঠিক পথ (Route) নির্ধারণ করে।

## ✅ Exam Ready Answer (English)

A Router is a networking device that connects different networks and forwards data packets based on IP addresses.

### Easy Example

Your home Wi-Fi Router connects your Home Network to the Internet.

### Key Points

- Layer: Network Layer (Layer 3)

- Uses IP Address

- Connects Different Networks

# 4. BRIDGE

## 🟢 বাংলা Concept

Bridge একই ধরনের দুটি LAN Segment-কে সংযুক্ত করে এবং অপ্রয়োজনীয় Traffic কমায়।

## ✅ Exam Ready Answer

A Bridge is a networking device that connects two similar LAN segments and filters traffic using MAC addresses.

### Easy Example

Two office LANs connected through a Bridge.

### Key Points

- Layer: Data Link Layer (Layer 2)

- Filters Traffic

# 5. REPEATER

## 🟢 বাংলা Concept

দূরে Data পাঠাতে গেলে Signal দুর্বল হয়ে যায়।

Repeater সেই দুর্বল Signal-কে আবার শক্তিশালী (Regenerate) করে।

## ✅ Exam Ready Answer (English)

A Repeater is a networking device that regenerates weak signals to extend the transmission distance.

### Easy Example

Long Optical Fiber Cable

### Key Points

- Layer: Physical Layer

- Regenerates Signal

# 6. MODEM

## 🟢 বাংলা Concept

Modem Digital Signal-কে Analog এবং Analog Signal-কে Digital-এ রূপান্তর করে।

(Modulator + Demodulator)

## ✅ Exam Ready Answer (English)

A Modem is a device that converts digital signals into analog signals and analog signals into digital signals for Internet communication.

### Easy Example

Broadband Internet Connection

# 7. GATEWAY

## 🟢 বাংলা Concept

ভিন্ন ধরনের Network বা Protocol-এর মধ্যে যোগাযোগ করায়।

## ✅ Exam Ready Answer (English)

A Gateway is a networking device that connects two different networks using different communication protocols.

### Easy Example

A Bank System communicating with a Government Server using different protocols.

# 8. ACCESS POINT (AP)

## 🟢 বাংলা Concept

Wireless Device-কে LAN-এর সাথে সংযুক্ত করে।

## ✅ Exam Ready Answer (English)

An Access Point (AP) is a networking device that allows wireless devices to connect to a wired network.

### Easy Example

Office Wi-Fi

# 9. FIREWALL ⭐⭐⭐

## 🟢 বাংলা Concept

Firewall Network-এর Security Guard-এর মতো কাজ করে।

এটি ক্ষতিকর Traffic Block করে এবং অনুমোদিত Traffic Allow করে।

# Network

## 🟢 বাংলা Concept

Network হলো দুই বা ততোধিক Computer, Server, Printer, Router বা অন্যান্য Device-কে Cable অথবা Wireless-এর মাধ্যমে সংযুক্ত করার ব্যবস্থা, যাতে তারা Data, Information এবং Resources Share করতে পারে।

## ✅ Exam Ready Answer (English)

A Computer Network is a collection of two or more interconnected computers or devices that communicate and share data, resources, and services through communication links.

### Easy Example

Suppose all computers of Pubali Bank are connected so that every branch can access customer accounts and process transactions securely. This is called a Computer Network.

# Network Topology

## 🟢 বাংলা Concept

Network Topology হলো একটি Network-এর সকল Device (Computer, Switch, Hub, Router, Server)-এর Physical অথবা Logical Arrangement।

সহজভাবে,

Network = Devices Connected

Topology = Devices কীভাবে Connected আছে।

## ✅ Exam Ready Answer (English)

Network Topology is the physical or logical arrangement of computers, networking devices, and communication links in a computer network.

It defines how devices are connected and how data is transmitted between them.

# 1. BUS TOPOLOGY

### Figure

BUS TOPOLOGY

[T]===============================================[T]

|              |              |        |

[PC1]          [PC2]          [PC3]   [Printer]

<------ Data travels on Backbone ------>

T = Terminator

# Bus Topology

## 🟢 বাংলা Concept

Bus Topology হলো এমন একটি Network Topology যেখানে সব Computer বা Device একটি মাত্র Main Communication Cable (Backbone Cable)-এর সাথে সংযুক্ত থাকে।

যখন কোনো Computer Data পাঠায়, তখন Data Backbone Cable-এর মাধ্যমে সব Computer-এর কাছে Broadcast হয়। এরপর প্রত্যেক Computer Destination MAC Address পরীক্ষা করে। যার MAC Address মিলে, শুধুমাত্র সেই Computer Data গ্রহণ (Accept) করে; অন্যরা Data উপেক্ষা (Discard) করে।

Backbone Cable-এর দুই প্রান্তে Terminator থাকে, যা Signal-এর প্রতিফলন (Signal Reflection) প্রতিরোধ করে।

## ✅ Exam Ready Answer (English)

Bus Topology is a network topology in which all computers or devices are connected to a single communication cable called the Backbone Cable.

When a device sends data, it is broadcast through the backbone cable to all connected devices. Each device checks the destination MAC address in the data frame. Only the device whose MAC address matches the destination accepts the data, while all other devices discard it.

A Terminator is placed at both ends of the backbone cable to absorb signals and prevent signal reflection, ensuring reliable communication.

### Advantages

- Low Cost

- Easy Installation

- Less Cable Required

### Disadvantages

- Backbone Failure → Entire Network Stops

### Example

Small Office Network

[T]===============================================[T]

Backbone Cable (Main Bus Cable)

│             │             │             │

[PC-1] ========> [PC-2] ======> [PC-3] ====> [PC-4]

Ignore         Accept      Ignore

2. STAR TOPOLOGY ⭐⭐⭐ (Most Star Topology-তে সব Computer একটি Central Switch/Hub-এর সাথে সংযুক্ত থাকে।

যখন একটি Computer অন্য Computer-কে Data পাঠায়, তখন Data প্রথমে Switch-এর কাছে যায়। Switch Destination MAC Address দেখে শুধুমাত্র নির্দিষ্ট Computer-এর কাছে Data পাঠায়।

এজন্য অপ্রয়োজনীয় Computer Data পায় না এবং Network দ্রুত (Fast) ও নিরাপদ (Efficient) হয়।

## ✅ Exam Ready Answer (English)

Star Topology is a network topology in which all devices are connected to a central Switch or Hub.

When a device sends data, it first reaches the central switch. The switch checks the destination MAC address and forwards the data only to the intended destination device.

### Figure

STAR TOPOLOGY

[PC1]

│

│

[PC2] ─────────────── [ SWITCH ] ─────────────── [PC3]

│

│

[Server]

│

[PC4]

### Figure Explanation

- Switch → Central Device

- PCs → Connected Devices

- All communication passes through the Switch.

### Advantages

- Easy Management

- High Performance

- Easy Troubleshooting

- Easy Expansion

### Disadvantages

- Switch Failure → Entire Network Stops

### Example

Banks, Offices, Universities

# 3. RING TOPOLOGY

Ring Topology-তে Data একটি নির্দিষ্ট দিক (Clockwise বা Counterclockwise) অনুসরণ করে একটি Computer থেকে পরবর্তী Computer-এ যেতে থাকে।

প্রত্যেক Computer Data Frame-এর Destination MAC Address পরীক্ষা করে।

- যদি MAC Address মিলে, তাহলে সেই Computer Data Accept করে।

- যদি না মিলে, তাহলে Data পরবর্তী Computer-এর কাছে Forward করে দেয়।

এভাবে Data Destination-এ পৌঁছানো পর্যন্ত Ring-এর মধ্যে ঘুরতে থাকে।

# ✅ Exam Ready Answer (English)

In a Ring Topology, data travels in a single direction (clockwise or counterclockwise) from one device to the next around the ring.

Each device receives the data frame and checks the destination MAC address.

- If the destination MAC address matches its own MAC address, the device accepts the data.

- Otherwise, it forwards the frame to the next device in the ring.

This process continues until the data reaches the intended destination.

### Easy Example

Suppose PC1 wants to send data to PC3.

The data travels as follows:

PC1 → PC2 → PC3

- PC2 checks the destination MAC address. Since it does not match, it forwards the data.

- PC3 finds that the destination MAC address matches its own MAC address and accepts the data.

### Figure

RING TOPOLOGY

[PC1] -------- [PC2]

|               |

|               |

[PC4] -------- [PC3]

Data Flow →

### Advantages

- No Data Collision

- Equal Access

### Disadvantages

- One Device Failure may affect the entire Network.

### Example

Industrial Control Network

# 4. MESH TOPOLOGY

## বাংলা Concept

Mesh Topology-তে প্রতিটি Computer অন্য সব Computer-এর সাথে Direct Connection রাখে।

তাই Data কোনো মধ্যবর্তী Device ছাড়াই সরাসরি Destination-এ যায়।

Destination Computer MAC Address মিলিয়ে Data গ্রহণ করে।

## ✅ Exam Ready Answer (English)

In Mesh Topology, every device has a direct connection with every other device.

Data travels directly to the destination without passing through intermediate devices.

The destination device checks the destination MAC address and accepts the data.

### Easy Example

PC1 ───────── PC4

PC1 directly sends data to PC4.

No Switch

No Hub

No Broadcast

### Figure

MESH TOPOLOGY

[PC1]-------------[PC2]

|\             /|

| \           / |

|  \         /  |

|   \       /   |

|    \     /    |

|     \   /     |

|      \ /      |

[PC3]-------------[PC4]

⭐ এখানে প্রত্যেক PC অন্য সব PC-এর সাথে Connected।

### Figure Explanation

- Every Device ↔ Every Device

- Multiple Communication Paths

### Advantages

- Highly Reliable

- High Security

### Disadvantages

- Expensive

- Complex

### Example

Military Network, Data Center

# 5. TREE TOPOLOGY

### 🟢 বাংলা Concept

Tree-এর মতো Layer বা Branch আকারে Connected থাকে।

## ✅ Exam Ready Answer

Tree Topology is a hierarchical topology that combines the characteristics of Bus and Star Topologies.

### Figure

TREE TOPOLOGY

[Core Switch]

|

-------------------------

|                       |

[Switch-1]             [Switch-2]

/      \              /      \

[PC1]   [PC2]         [PC3]   [PC4]

⭐ এটি একটি Tree (গাছ) এর মতো Root এবং Branch তৈরি করেছে।

### Figure Explanation

- Core Switch → Root

- Switch-1, Switch-2 → Branch

- PCs → Leaf Nodes

### Advantages

- Easy Expansion

- Suitable for Large Networks

### Disadvantages

- Core Failure affects many devices.

### Example

University Campus Network

# 6. HYBRID TOPOLOGY

### 🟢 বাংলা Concept

দুই বা ততোধিক Topology-এর সমন্বয়ে Hybrid Topology তৈরি হয়।

## ✅ Exam Ready Answer

Hybrid Topology is a combination of two or more different network topologies within a single network.

### Figure

HYBRID TOPOLOGY

★ STAR NETWORK - 1

[Switch-1]

/     |      \

[PC1]  [PC2]   [PC3]

================== BACKBONE CABLE ==================

★ STAR NETWORK - 2

[Switch-2]

/     |      \

[PC4]  [PC5]   [PC6]

Hybrid = Star + Bus

⭐ এখানে দুইটি Star Network একটি Backbone Cable (Bus) দিয়ে যুক্ত হয়েছে, তাই এটি Hybrid Topology।

### Figure Explanation

- Switch-1 → First Star Network

- Switch-2 → Second Star Network

- Backbone Cable → Connects Both Star Networks

### Advantages

- Flexible

- Scalable

- Reliable

### Disadvantages

- Expensive

- Complex Design

### Example

Large Banks, Corporate Offices

# ⭐ Exam Comparison (Very Important)

| Topology | Shape | Main Device | Failure Effect | Used In |
| --- | --- | --- | --- | --- |
| Bus | Straight Line | Backbone Cable | Backbone Failure → Whole Network Down | Small Office |
| Star | ⭐ Star | Switch/Hub | Switch Failure → Whole Network Down | Banks |
| Ring | ⭕ Circle | No Central Device | One Failure affects Ring | Industry |
| Mesh | Web/Net | No Central Device | Very Reliable | Military |
| Tree | 🌳 Tree | Core Switch | Core Failure affects Branches | University |
| Hybrid | Mixed | Multiple Devices | Depends on Design | Large Banks |

# ✅ Example-1 (Class A)

### Given:

IP Address = 10.50.20.100

Subnet Mask = 255.224.0.0

# Step-1: Identify the Class

প্রথম Octet = 10

Class Range

| Class | Range |
| --- | --- |
| A | 1–126 |
| B | 128–191 |
| C | 192–223 |

👉 10 ⇒ Class A

# Step-2: Default Subnet Mask

Class A-এর Default Mask

255.0.0.0

Binary

11111111.00000000.00000000.00000000

1-এর সংখ্যা

8

তাই Prefix

/8

# Step-3: Given Mask

255.224.0.0

224-এর Binary

11100000

পুরো Binary

11111111.11100000.00000000.00000000

1-এর সংখ্যা

8 + 3 = 11

অতএব

255.224.0.0 = /11

# Step-4: Borrowed Bits

Default ছিল

/8

Given হলো

/11

Borrowed Bits

11 − 8 = 3 Bits

# Step-5: Remaining Host Bits

Class A-এর Default Host Bits

24

Borrow করেছে

3

Remaining

24 − 3 = 21 Bits

# Step-6: Number of Subnets

Formula

2^(Borrowed Bits)

2³ = 8

✅ 8 Subnets

# Step-7: Number of Hosts

Formula

2^(Host Bits) − 2

2²¹ − 2

= 2097152 − 2

= 2097150

✅ 2,097,150 Usable Hosts

# Step-8: Block Size

Formula

256 − 224

32

যেহেতু 224 দ্বিতীয় Octet-এ, তাই Block Size দ্বিতীয় Octet-এ হবে।

Network IDs

0

32

64

96

128

160

192

224

IP-এর দ্বিতীয় Octet

50

50 কোন Range-এ?

32–63

# Step-9: Network Address

10.32.0.0

Broadcast Address

10.63.255.255

Valid Hosts

10.32.0.1

↓

10.63.255.254

# Final Answer (Class A)

| Item | Answer |
| --- | --- |
| Class | A |
| Default Mask | 255.0.0.0 (/8) |
| Given Mask | 255.224.0.0 (/11) |
| Borrowed Bits | 3 Bits |
| Host Bits | 21 Bits |
| Subnets | 8 |
| Usable Hosts | 2,097,150 |
| Block Size | 32 |
| Network Address | 10.32.0.0 |
| Broadcast Address | 10.63.255.255 |
| Valid Hosts | 10.32.0.1 – 10.63.255.254 |

# ✅ Example-2 (Class B)

### Given:

IP Address = 172.16.35.100

Subnet Mask = 255.255.240.0

# Step-1: Identify the Class

প্রথম Octet = 172

Range

128–191

👉 Class B

# Step-2: Default Subnet Mask

255.255.0.0

Binary

11111111.11111111.00000000.00000000

1-এর সংখ্যা

8 + 8 = 16

তাই Prefix

/16

# Step-3: Given Mask

255.255.240.0

240-এর Binary

11110000

পুরো Binary

11111111.11111111.11110000.00000000

1-এর সংখ্যা

8 + 8 + 4 = 20

অতএব

255.255.240.0 = /20

# Step-4: Borrowed Bits

Default

/16

Given

/20

Borrowed

20 − 16 = 4 Bits

# Step-5: Remaining Host Bits

Class B-এর Host Bits

16

Borrow করেছে

4

Remaining

16 − 4 = 12 Bits

# Step-6: Number of Subnets

Formula

2^(Borrowed Bits)

2⁴ = 16

✅ 16 Subnets

# Step-7: Number of Hosts

Formula

2^(Host Bits) − 2

2¹² − 2

= 4096 − 2

= 4094

✅ 4094 Usable Hosts

# Step-8: Block Size

Formula

256 − 240

16

Network IDs

0

16

32

48

64

80

96

112

IP-এর তৃতীয় Octet

35

35 কোন Range-এ?

32–47

# Step-9: Network Address

172.16.32.0

Broadcast Address

172.16.47.255

Valid Hosts

172.16.32.1

↓

172.16.47.254

# Final Answer (Class B)

| Item | Answer |
| --- | --- |
| Class | B |
| Default Mask | 255.255.0.0 (/16) |
| Given Mask | 255.255.240.0 (/20) |
| Borrowed Bits | 4 Bits |
| Host Bits | 12 Bits |
| Subnets | 16 |
| Usable Hosts | 4094 |
| Block Size | 16 |
| Network Address | 172.16.32.0 |
| Broadcast Address | 172.16.47.255 |
| Valid Hosts | 172.16.32.1 – 172.16.47.254 |

# ⭐ তিনটি Class-এর একসাথে Comparison (সবচেয়ে গুরুত্বপূর্ণ)

| Item | Class A | Class B | Class C |
| --- | --- | --- | --- |
| IP Example | 10.50.20.100 | 172.16.35.100 | 192.168.10.25 |
| Default Mask | 255.0.0.0 | 255.255.0.0 | 255.255.255.0 |
| Default Prefix | /8 | /16 | /24 |
| Default Host Bits | 24 | 16 | 8 |
| Given Prefix | /11 | /20 | /27 |
| Borrowed Bits | 3 | 4 | 3 |
| Remaining Host Bits | 21 | 12 | 5 |
| Subnets | 8 | 16 | 8 |
| Usable Hosts | 2,097,150 | 4,094 | 30 |

# Example (Class C)

## Given:

IP Address = 192.168.10.25

Subnet Mask = 255.255.255.224

# Step-1: Identify the Class

প্রথম Octet = 192

Class Range:

| Class | Range |
| --- | --- |
| A | 1–126 |
| B | 128–191 |
| C | 192–223 |

👉 192 ⇒ Class C

# Step-2: Write the Default Subnet Mask

Class C-এর Default Mask:

255.255.255.0

Binary:

11111111.11111111.11111111.00000000

1-এর সংখ্যা

8+8+8=24

তাই Prefix

/24

# Step-3: Convert Given Mask into Binary

Given Mask

255.255.255.224

Binary

255 = 11111111

255 = 11111111

255 = 11111111

224 = 11100000

সবগুলো একসাথে

11111111.11111111.11111111.11100000

1-এর সংখ্যা

8+8+8+3=27

অতএব

255.255.255.224 = /27

# Step-4: Borrowed Bits

Default ছিল

/24

এখন

/27

Borrow করেছে

27−24=3 Bits

👉 Host Portion থেকে 3 Bits Borrow করা হয়েছে।

# Step-5: Remaining Host Bits

Class C-তে Host Bits ছিল

8 Bits

Borrow করেছে

3 Bits

Remaining

8−3=5 Bits

# Step-6: Number of Subnets

Formula

2^(Borrowed Bits)

2³=8

✅ 8 Subnets

# Step-7: Number of Hosts

Formula

2^(Host Bits)-2

2⁵−2

32−2

30

✅ 30 Usable Hosts

# ❓এখানে "-2" কেন?

এটি 2 Bits না, 2টি Host Address।

অনেকে ভুল করে বলে "২টা Bit Minus করি"।

❌ এটা ভুল।

সঠিক কথা হলো—

আমরা ২টি Address ব্যবহার করতে পারি না।

কারণ—

### প্রথম Address = Network Address

উদাহরণ

192.168.10.0

এটি পুরো Network-কে বোঝায়।

কোনো Computer-কে দেওয়া যায় না।

### শেষ Address = Broadcast Address

192.168.10.31

এটি পুরো Network-এ Message পাঠানোর জন্য Reserved।

এটিও কোনো Host-কে দেওয়া যায় না।

তাই

মোট Address

32

কিন্তু

1 Network Address

+

1 Broadcast Address

Reserved।

তাই

32−2=30

👉 Minus 2 মানে 2টি Address, Bit নয়।

# Step-8: Block Size

Formula

256−224

32

Block Size

32

Network Numbers হবে

0

32

64

96

128

160

192

224

# Step-9: Network Address

শেষ Octet

25

25 কোন Range-এ?

0----31

অতএব

Network

192.168.10.0

Broadcast

192.168.10.31

Valid Hosts

192.168.10.1

↓

192.168.10.30

# Final Answer

| Item | Answer |
| --- | --- |
| Class | C |
| Default Mask | 255.255.255.0 (/24) |
| Given Mask | 255.255.255.224 (/27) |
| Borrowed Bits | 3 Bits |
| Remaining Host Bits | 5 Bits |
| Number of Subnets | 8 |
| Number of Usable Hosts | 30 |
| Block Size | 32 |
| Network Address | 192.168.10.0 |
| Broadcast Address | 192.168.10.31 |
| Valid Host Range | 192.168.10.1 – 192.168.10.30 |

# OSI (Open Systems Interconnection) Model

## 🟢 Concept (বাংলা)

OSI Model হলো ISO কর্তৃক তৈরি একটি 7-layer Reference Model, যা দেখায় একটি Computer থেকে অন্য Computer-এ Data কীভাবে ধাপে ধাপে আদান-প্রদান হয়। প্রতিটি Layer-এর নির্দিষ্ট কাজ রয়েছে এবং প্রতিটি Layer উপরের Layer-কে Service প্রদান করে।

## ✅ Exam Ready Answer (English)

The OSI (Open Systems Interconnection) Model is a seven-layer reference model developed by the International Organization for Standardization (ISO). It standardizes network communication by dividing the communication process into seven logical layers, where each layer performs a specific function and provides services to the layer above it.

# OSI Model (Graphical View)

OSI MODEL

User

│

▼

┌──────────────────────────────────────────────┐

│ 7. APPLICATION                               │

│ Services for End Users                       │

│ Protocols: HTTP, HTTPS, FTP, SMTP, DNS       │

│ PDU : Data                                   │

└──────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────┐

│ 6. PRESENTATION                              │

│ Encryption, Decryption, Compression          │

│ Protocol: SSL/TLS                            │

│ PDU : Data                                   │

└──────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────┐

│ 5. SESSION                                   │

│ Creates, Maintains & Ends Sessions           │

│ Protocols: RPC, NetBIOS                      │

│ PDU : Data                                   │

└──────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────┐

│ 4. TRANSPORT                                 │

│ Reliable Delivery, Segmentation              │

│ Flow Control, Error Recovery                 │

│ Protocols: TCP, UDP                          │

│ Address : Port Number                        │

│ PDU : Segment                                │

└──────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────┐

│ 3. NETWORK                                   │

│ Routing & Logical Addressing                 │

│ Device : Router                              │

│ Protocols: IP, ICMP                          │

│ Address : IP Address                         │

│ PDU : Packet                                 │

└──────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────┐

│ 2. DATA LINK                                 │

│ Framing, MAC Address, Error Detection        │

│ Devices : Switch, Bridge                     │

│ Address : MAC Address                        │

│ PDU : Frame                                  │

└──────────────────────────────────────────────┘

│

▼

┌──────────────────────────────────────────────┐

│ 1. PHYSICAL                                  │

│ Sends Raw Bits Through Cable/Fiber/Wireless  │

│ Devices : Hub, Repeater                      │

│ PDU : Bits                                   │

└──────────────────────────────────────────────┘

│

▼

Physical Medium

# Data Flow (Very Important)

## Sender Side (Encapsulation)

Application

│

Presentation

│

Session

│

Transport   → Segment

│

Network     → Packet

│

Data Link   → Frame

│

Physical    → Bits

│

Transmission Media

## Receiver Side (Decapsulation)

Transmission Media

│

Physical    → Bits

│

Data Link   → Frame

│

Network     → Packet

│

Transport   → Segment

│

Session

│

Presentation

│

Application

# PDU (Protocol Data Unit)

| Layer | PDU |
| --- | --- |
| Application | Data |
| Presentation | Data |
| Session | Data |
| Transport | Segment |
| Network | Packet |
| Data Link | Frame |
| Physical | Bits |

# Address Used

| Layer | Address |
| --- | --- |
| Transport | Port Number |
| Network | IP Address |
| Data Link | MAC Address |

# Networking Devices

| Device | Layer |
| --- | --- |
| Hub | Physical |
| Repeater | Physical |
| Switch | Data Link |
| Bridge | Data Link |
| Router | Network |
| Gateway | Application / Multiple Layers |

# Easy Banking Example

Suppose a customer transfers ৳50,000 using the National Bank Mobile App.

Application

User clicks Transfer

↓

Presentation

Encrypts the data (SSL/TLS)

↓

Session

Maintains the connection

↓

Transport

TCP ensures reliable delivery

↓

Network

Uses IP Address to reach the Bank Server

↓

Data Link

Uses MAC Address within the local network

↓

Physical

Sends Bits through Wi-Fi/Fiber Cable

# Easy Banking Example

Suppose a customer transfers ৳10,000 using the Pubali Bank Mobile App.

- Application Layer → User requests the transfer.

- Presentation Layer → Encrypts the data using SSL/TLS.

- Session Layer → Establishes and manages the session.

- Transport Layer → Uses TCP to ensure reliable delivery.

- Network Layer → Uses the destination IP address to route the packet.

- Data Link Layer → Uses the destination MAC address to deliver the frame within the local network.

- Physical Layer → Sends the data as bits through Wi-Fi, Ethernet, or Fiber.

# Advantages of OSI Model

- Standardizes network communication.

- Simplifies network design and troubleshooting.

- Provides modular architecture.

- Ensures interoperability between different vendors.

- Makes maintenance and upgrades easier.

# Disadvantages of OSI Model

- More theoretical than practical.

- Complex compared to the TCP/IP model.

- Some layers have overlapping functions.